Agricultural Industry Standard of the People’s Republic of China

NY/T 8-2006

Replace NY/T 8-1984

Thermal performance test method for civil firewood stoves

Issued date 2006-07-10

Implementation date 2006-10-01

Issued by Ministry of Agriculture

Foreword

This standard is the revision of NY/T 8-1984 "Thermal performance test method for civil firewood stoves".

The main changes are as follows compared to NY/T 8-1984:

——Change the standard name to "Thermal performance test method for civil firewood stoves";

——Add the definition and calculation method of the heat supply intensity in the calefaction section;

——Add the definition and calculation method of the heat supply intensity in the evaporation section;

——Add the definition of the rated power;

——Remove the content of rebound speed;

——Add the cooking performance test.

Annex A, B and C are informative annex.

This standard is under the jurisdiction of Ministry of Agriculture.

This standard was drafted by: Energy and Environmental Protection Technology Development Center of Ministry of Agriculture.

The main drafters: Tian Yishui, Cui Yuanbo, Guo Jiye, Gao Yuanchao and Wu Wenshu.

The previous versions of this standard:

——NY/T 8-1984.

Thermal performance test method for civil firewood stoves

1	Scope

This standard specifies the test method for thermal performance of civil firewood stoves.

This standard applies to the single or multiple civil stoves using solid biomass as the fuel, such as the firewood, straw or animal waste.

2	Normative references

The following standards contain provisions which, through reference in this text, constitute provisions of this standard. For dated reference, subsequent amendments to (excluding correction to), or revisions of, any of these publications do not apply. However, the parties to agreements based on this standard are encouraged to investigate the possibility of applying the most recent editions of the standards. For undated references, the latest edition of the normative document referred to applies.

NY/T 12-1985 Test method of calorific value of biomass fuel (original GB 5186-85)

3 	Terms and definitions

The following terms and definitions apply to this standard.

3.1	calefactive heat intensity

During the time from the beginning of the burning to the time when the water reaches the boiling point, the heat absorbed by the water in the boiler in the unit time. It indicates the starting performance of the civil firewood stoves.

3.2	vaporizing heat intensity

The heat absorbed by the water in the boiler in the unit time during the evaporation stage. It indicates the continuous heating performance of the civil firewood stoves.

3.3	rated power

The heating intensity of the civil firewood stoves during the evaporation stage

3.4	thermal efficiency

The percentage of the heat absorbed by the water during the calefactive and evaporation stage to the total caloric value of the firewood. It indicates the degree of the heat utilization of the civil firewood stoves.

3.5	kitchen performance test

Test method of heat performance for the civil firewood stoves in the ordinary house environment. It is suitable for the single or multiple civil stoves use solid biomass as fuels such as firewood, straw and animal waste. It is convenient to compare the heat performance of firewood stoves in the ordinary house environment, to know the actual energy saving performance and evaluate the economic benefit.

Note: Refer to the provisional international standards of stove efficiency test, VITA, United States, 1992.

3.6	specific daily consumption

The daily firewood heat consumption per person. It indicates the cooking heat performance of the civil stoves.

4 Test instruments and equipment

4.1Test instruments and equipment are shown in Table l.

Table 1	Test instruments and equipment

| No. | Name | Measuring range and resolution | Quantity |
| --- | --- | --- | --- |
| 1 | Thermometer | Measuring range 0°C～150°C, resolution of 0.2°C | One |
| 2 | Clock | daily difference less than 1min | one |
| 3 | Platform scale | Measuring range 0kg～10kg, resolution of 0.005kg | one |
| 4 | Psychrometer |  | one |
| 5 | Anemometer | Measuring range 0m/s～10m/s, resolution of 0.5m/s | one |

4 .2	Choose the aluminum or iron boiler according to the parameters in Table 2

Table 2	Test parameters

| No. | Rated power kW | Initial water weight GS1, kg | Aluminum boiler diameter DL, mm | Iron boiler diameter DT, mm | Firewood weight GC, kg |
| --- | --- | --- | --- | --- | --- |
| 1 | 3.5 | 6 | 240 | 440 | 2.0 |
| 2 | 3.5～7.0 | 10 | 520 | 520 | 2.0～4.0 |
| 3 | >7.0 | 15 | 320 | 600 | >4.0 |

5	Test conditions and preparation

5.1	Test conditions

Environmental temperature: 10°C~30°C;

Relative humidity: less than 85%;

Wind speed: less than 1.0 m/s;

The spacing among the stoves should be greater than 1.5m and the stoves should face the same direction.

5.2	Preparation

5.2.1 The instruments should be calibrated and valid and marked with the certificate issued by the legal metrology. Check the instruments before use.

5.2.2 Determine the firewood usage Gc (Table 2) according to the rated power of the stoves. The firewood should be able to allow the stove to work under the rated power state for 0.5h.

5.2.3 Measure the natural dry firewood according to Gc.

5.2.4 Measure the receive-based low level calorific value of firewood according to NY/T 12.

6	Test requirements

The heat performance test of civil firewood stoves twice and calculate the average results of the two tests. The difference of thermal efficiency values of two tests should be less than 5%.

7	Test procedure

7.1	Measure water of Gs and put in the boiler, measure the initial temperature t1 and cover the lid.

7.2	Ignite and record the start time of burning T1.

7.3	Add the firewood continuously. When the water reaches the boiling point, open the lid and record the water temperature t2 and the time T2.

7.4	Continue to add firewood to make the water evaporate until ran out of firewood. Record the time T3 when the water temperature is 2°C below the boiling point and measure the weight of remaining water Gs3.

7.5	Record the electricity quantity E for the stoves using fans.

8	Test results and calculation

8.1	calefactive heat intensity

Wherein:

Pi		calefactive heat intensity in (kW);

4.18		specific heat capacity of water at constant pressure [kJ/(kg·°C)];

Gs1		initial water weight, (kg)

t2		boiling temperature of water, °C;

t1		initial temperature of water, °C;

T2 - T1	difference of time when boiling and time of ignition, min.

8.2 	vaporizing heat intensity

Wherein:

P2		vaporizing heat intensity (kW)

Gs2		water weight when reaches boiling point, could use the initial water weight Gs1, (kg)

Gs3		water weight when reaches 2 °C below the boiling point, kg;

γ		water latent vaporization heat at the boiling point, see Annex B, kJ/kg;

T1 - T2	difference of time when boiling and time when the water reaches 2°C below the boiling point, min.

8.3	Thermal efficiency

8.3.1 single stove thermal efficiency

Wherein:

η		single stove thermal efficiency;

Gc		firewood weight, kg;

receive-based low level calorific value, kJ/kg, refer to the Annex C when in limited test conditions.

8.3.2 multiple stoves thermal efficiency

Wherein:

n		number of stoves;

i		one of the stoves;

GS1i		initial water weight of the ith boiler, kg;

t2i		boiling point temperature of the ith boiler, °C;

t1i		initial temperature of the ith boiler, °C;

GS2i  	water weight when at the boiling point, could use the initial water weight GS1i，kg:

GS3i  	water weight when at 2°C below the boiling point, kg.

8.4	specific daily consumption

specific daily consumption should be determined in the cooking performance test. See Annex A for the test method and calculation method.

9	Test report

9.1	Test report cover should include:

a) civil stove name;

b) model and specification;

c) manufacture (research) unit;

d) test location;

e) test date;

f) test unit;

g) test principal;

h) tester;

i) fuel analysis units.

9 .2	Test report body should include:

a) test objectives and requirements;

b) test instruments and equipment;

c) test conditions: environmental temperature, relative humidity and wind speed;

d) original test records (see Table 3)

e) summarized table of test results (see Table 4)

9.3	The original data and test reports should be archived for future reference.

Table 3 original test records

| No. | Item | Symbol | Unit | Test data I | Test data II |
| --- | --- | --- | --- | --- | --- |
| 1 | firewood weight | Gc | kg |  |  |
| 2 | receive-based low level calorific value of the firewood | Qnet,v, ar | kJ/kg |  |  |
| 3 | initial water weight | GS1 | kg |  |  |
| 4 | initial temperature | t1 | °C |  |  |
| 5 | ignition time | T1 |  |  |  |
| 6 | boiling temperature | t2 | °C |  |  |
| 7 | boiling time | T2 |  |  |  |
| 8 | time when 2°C below the boiling point | T3 |  |  |  |
| 9 | remaining water weight | GS3 | kg |  |  |
| 10 | electric usage | E | kW•h |  |  |

Table 4 summarized table of test results

| Test No. | calefactive heat intensity kW | vaporizing heat intensity kW | Thermal efficiency% |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| average |  |  |  |

Annex A

(Informative)

Cooking performance test

A.1	Test instruments and equipment

Test instruments and equipment are shown in Table 1.

A.2	Test preparation

A.2.1 Test principal should be familiar with this standard and has professional experience. Test outline should be determined according to the relevant requirements and specific circumstance, including:

a)test task and requirements;

b)test items;

c)test schedule.

A.2.2 Choose the families to participate in the test and number the families.

A.2.3 Determine the test date for 7 consecutive days.

A.2.4 According to the cooking requirements of each family, prepare the required firewood for 7 days test. The firewood should be in line with the local situation.

A.2.5 Test instruments used in the test should be valid and calibrated, marked with the certificate issued by department of legal metrology. Check the instruments before tests.

A.2.6 Determine the receive-based low calorific value of the firewood according to the requirements of NY/T 12.

A.3	Test requirements

A.3 .1 Choose families of roughly the same economic level. The test should include at least 5 families.

A.3 .2 No changes of the family members during the test and the main food species should be substantially the same and no dramatic change in fruit and vegetable varieties.

A.4 Test items

A.4.1 Measure the total firewood consumption of each family in the test Gci.

A.4.2 Statistics of dining member count, age and gender of each family.

A.5	Test result and calculation

A.5.1	standard number of adults

Wherein:

Ri		standard number of adults of the ith family, in person;

i		the ith family;

4		number of sex age sections;

j		the jth sex age section;

λj		conversion rate of standard number of adults for jth sex age section, see Table A.1;

Rij		standard number of adults for ith family and jth sex age section, in person.

Table A.1standard number of adults according to the sex age section

| No. | Item | Symbol | Conversion rate |
| --- | --- | --- | --- |
| 1 | child of 0～14 | λ1 | 0.5 |
| 2 | woman above 14 | λ2 | 0.8 |
| 3 | man of 15～59 | λ3 | 1.0 |
| 4 | man above 59 | λ4 | 0.8 |
| Note: determined according to the simplified version of League of Nations formula (Wood fuel investigation guide, Keirth Openshaw,1987)。 |  |  |  |

A.5.2	specific daily consumption

Wherein:

Pi		specific daily consumption of ith family, in [kJ/(person·d)];

Gci		total firewood consumption of ith family in the test, in kg;

Qnet,v,ar	receive-based low calorific value of the firewood, in kJ/kg (refer to Annex C if in limited test environment);

7		test days, in d,

A.5.3	average specific daily consumption

Wherein:

average specific daily consumption per member, in [kJ/(person·d)];

n	total number of families in the test.

A.5.4	standard deviation

Wherein;

σ	standard deviation, in [kJ/(person·d)]。

A.5.5	standard deviation of average specific daily consumption

Wherein:

σP	standard deviation of average specific daily consumption per member, in [kJ/(person·d)]。

A.6	Test report

A.6.1 Test report cover should include:

a) civil stove name;

b) model and specification;

c) manufacture (research) unit;

d) test location;

e) test date;

f) test unit;

g) test principal;

h) tester;

i) fuel analysis units.

A.6.2 Test report body should include:

a) test objectives and requirements;

b) test instruments and equipment;

c) test conditions: environmental temperature, relative humidity and wind speed;

d) original test records (see Table A.2)

e) summarized table of test results (see Table A.3)

A.6.3 The original data and test reports should be archived for future reference.

Table A.2 original test records

| No. | Item | 1st | 2nd | 3rd | 4th | 5th |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Family name |  |  |  |  |  |
| 2 | Population of child of 0～14 |  |  |  |  |  |
| 3 | Population of woman above 14 |  |  |  |  |  |
| 4 | Population of man of 15～59 |  |  |  |  |  |
| 5 | Population of man above 59 |  |  |  |  |  |
| 6 | Total firewood consumption |  |  |  |  |  |

Table A.3 summarized table of test results

| No. | Item | Symbol | Unit | Results |
| --- | --- | --- | --- | --- |
| 1 | Receive-based low calorific value of the firewood | Qnet,v,ar | kJ/kg |  |
| 2 | Average specific daily consumption |  | kJ/(person·d) |  |
| 3 | standard deviation | σ | kJ/(person·d) |  |
| 4 | standard deviation of average specific daily consumption | σP | kJ/(person·d) |  |

Annex B

(Informative)

Latent heat of vaporization of water

Table B.1 gives latent heat of vaporization of water.

Table B.1 latent heat of vaporization of water

| No. | Boiling point, °C | latent heat of vaporization, kJ/kg |
| --- | --- | --- |
| 1 | 80 | 2304.4 |
| 2 | 82 | 2299.4 |
| 3 | 84 | 2294.4 |
| 4 | 86 | 2289.4 |
| 5 | 88 | 2284.0 |
| 6 | 90 | 2279.0 |
| 7 | 92 | 2273.5 |
| 8 | 94 | 2268.5 |
| 9 | 96 | 2263.5 |
| 10 | 98 | 2258.5 |
| 11 | 100 | 2253.0 |

Annex C

(Informative)

Simple test method of receive-based low calorific value of common solid biomass fuel

C.1 Sample 0.01kg firewood or straw in its store or location, shear as the samples.

C.2 Take certain amount of sample and bake to constant weight in the drying oven of 102°C～105°C. Use the reduced weight of sample to the original weight to calculate the moisture.

C.3 Get the receive-based low calorific value of the fuel according to the fuel type and calculated moisture content.

Table C.1 receive-based low calorific value of common solid biomass fuel, kJ/kg

moisture content percentage

type

maize straw, sorghum straw, cotton stalk, beanstalk, wheat straw, straw, willow twig, poplar twig, cow dung, masson pine, birch, basswood.

The data sources are from the agricultural engineering manual and rural energy engineering, Agriculture Press.